# Johanne Vigny , April 2025

# Zephyr RTOS

## Setup

### Prerequisites

#### Install

- Go to `C:\` to install zephyr in the directory `zephyrproject`

- Follow the Getting Started Guide  to install the Zephyr source code.

- Install `openocd` if needed or `jlink` depending on the runner needed by the board:

> choco install openocd

⚠️ For Jlink don’t forget to install the USB driver, otherwise it will not work.

#### Update

`west update`, use to keep your Zephyr repositories up to date, if zephyr is already installed.

Creating a Zephyr Application

- Create a project folder with the following structure:

<app>

├── CMakeLists.txt

├── prj.conf

├── src

|   └── main.c

└── boards

└── <board>.overlay or custom_board.dts

- Set the ZEPHYR_BASE environment variable to the Zephyr source code path:

set ZEPHYR_BASE=C:\zephyrproject\zephyr

- Active Python virtual environment.

C:\zephyrproject\.venv\Scripts\activate.bat

OR

- Alternatively, use the `env.cmd` script to set up the environment automatically. Make sure to update the script with the correct Zephyr source path.

⚠️ Run the above command every time you open a new terminal.

## Build and Flash the Project:

- `west build` to compile the project, ex: `west build -p always -b samd21_xpro app`

  - `-b <board>` ex: `-b samd21_xpro`:  Specifies the target board.

  - `-p` pristine: (optional)

    - `auto`

    - `always` => default option

    - `never`

  - Path to the <app> to build, if not specified use `.`

  - `-- -DDTC_OVERLAY_FILE=board.overlay` (optional): Specifies the path to an overlay file for the devicetree.

  - `-- -DBOARD_ROOT=<path>` (optional): Specifies the path to the `boards` directory where the custom board is located. Need to be an absolute path.

- `west flash` to flash the compile project to the board

  - `--build-dir <path>` or `-d` (optional): Specifies the directory to build the project

  - `--runners jlink` or `-r` (optional): Specifies the runners if supported by the board

## Other West command

- `west build -t menuconfig` or `west build -t guiconfig` to see or change Kconfig variables.

- `west config build.board <board>` sets permanently the default board if `-b` not specified

- `west debug` => attach debugger to the board, open `gdb`

  - `--runner openocd` or `-r`

## Vscode setting and extensions

### Extensions

- `ms-vscode.cpptools-extension-pack`		=> C/C++

- `ms-vscode.cmake-tools`				=> CMake

- `ms-vscode.vscode-serial-monitor`		=> Serial monitor

- `marus25.cortex-debug`				=> Embedded Debugger

- `nordic-semiconductor.nrf-devicetree`		=> Devicetree

### Configuration

#### Settings.json

- `"C_Cpp.default.compileCommands": <path>` after building the project add the path for the `compile_commands.json` in the build directory.

#### Tasks.json

Create tasks to build and flash the code on the board, in the tasks specify the env variable `ZEPHYR_BASE` to the zephyr code source, otherwise you will have a build error.

You can add input parameters if you want to dynamic parameter to the command run.

#### Launch.json

Create 2 launch configurations, one for launch and one for attach.

In each launch configuration add the following parameters:

- `name`		=> Name of the configuration

- `svdPath`		=> Path to svd file or ex: "Microchip::SAMD21_DFP@3.6.144"

- `deviceName`	=> Name of the board, need depending on the svd file, this setting is not recognise by Vscode but works.

- `device`		=> Name of the board

- `cwd`

- `executable`	=> Path to executable file .elf

- `request`		=> launch or attach

- `type`		=> cortex-debug

- `runToEntryPoint`	=> main function

- `servertype`		=> openocd, jlink ... or external to connect to an already existing session (with openocdm you want to connect to the debug session launch by `west flash`

- `gdbTarget`		=> Use only with serverType = external, define address to find the debugger server, ex: `:3333`

- `gdbPath`		=> path to gdb in zephyr sdk directory

- `preLaunchTask`	=> only for launch, command to run before debug, to build the project

- `configFiles`		=> for openocd, configuration file provides by zephyr

## Debug the Project

### Cortex Debug

Install the Vscode extension Cortex-Debug, then you need to create a `launch.json` file with 2 launch configuration. One to launch a debug session and one to attach a debug session. The launch session will build the project and flash it on the board then connects a debug session. The attach session will directly connect to the board executing code and open a debug session.

In the launch request you need to specify a `preLaunchTask` to build the project. So, you need to add a `tasks.json` file with a task to build the project.

Go to the onglet `Run and debug` in Vscode, then chose the configuration you want to use, and start debugging.

To access the register, you need to specify a `svdPath` in the `launch.json`, and if needed a `deviceName`.

### West debug

Run the command `west debug` in the terminal. It will open a `gdb` session.

Gdb Commands:

- `Ctlr + x`, then `2`		=> open the graphical interface

- `list`				=> with the graphical interface convert asm in C code

- `b <function>` 		=> create a break point on a function

- `b <file:line>` 		=> create a break point

- `b <address>`		=> create a break point

- `cont` or `c`			=> continue to run until the next break point or the end

- `next` or `n`			=> step over

- `step` or `s`			=> step in

- `backtrace` or `bt`		=> print callstack

- `print <var>` or `p <var>`	=> print variable, register…

- `info registers` or `i r`	=> print register value

## Devicetree

Use to define the hardware, all the peripheric.

### Overlay

File with the extension ` .overlay` are used to add peripheric or overwrite the devicetree of the original board.

Default location for devicetree when building the project:

- socs/<SOC>_<BOARD_QUALIFIERS>.overlay

- boards/<BOARD>.overlay

- boards/<BOARD>_<revision>.overlay

- <BOARD>.overlay

- app.overlay

Or use `-- -DDTC_OVERLAY_FILE=board.overlay` to explicitly define the path to the board in the `west build` command. It takes the priority over the default location define above.

You can add new node(device) to the devicetree or modify existing node.

To add node:

To modify existing node (label):

Depending on the `compatible` key each node will need different parameters, which can be found in the doc. It can be specific to the board used.

Led		=> `compatible = "gpio-leds"`

Button		=> `compatible = "gpio-keys"`

After building the project, the complete devicetree used will be in the file `<build directory>/zephyr/zephyr.dts`

### Custom Board

In your app directory `boards/<vendor>/<board_name>/` that will contain all the files describing the board. If no vendor name, it `other`. This directory will contain Kconfig and overlay to define the board, it’s also possible to define some code source with a CMakeLists.txt containing the following:

zephyr_library()

zephyr_library_sources(source_file.c)

## Kconfig

Used to enable or disable some features. The file `prj.conf` will contain the Kconfig at the app root. Kconfig will include the code source needed for your app.

- To enable Gpio: `CONFIG_GPIO=y`

- To enable Pwm: `CONFIG_PWM=y`

- …

After building the project, the complete kconfig used will be in the file `<build directory>/zephyr/.config`

## CMake

The `CMakeLists.txt` is located at the root of the app.

The default file contains:

Before pulling the project zephyr, you can define any variable you want.

To add other files in the project you can add a list of source files in `target_sources`, ex:

You can also add the path of the directory where to find header files with:

target_include_directories(app PRIVATE src)

Where `src` is a directory in the app project.

## Multiple configuration (Debug, Release …)

To have multiple configurations with different Kconfig or overlays, you need to define a variable in the CMakelists.txt file. This variable will be used to define the configuration used for the compilation. Depending on this configuration, you can then add the right Kconfig or overlay tree.

For example :

For this example, at the end of the west build command, you need to add the following flag : `-- -DBUILD_MODE=debug` where `BUILD_MODE` is the name of the variable defined.

## Power Management

It’s supported by zephyr for the board. Use the following Kconfig parameters to enable the use of low consumption:

- `CONFIG_PM`		=> enable power management when the board is idle

- `CONFIG_PM_DEVICE`	=> enable power management for devices.

- `CONFIG_PM_DEVICE_RUNTIME`

Then, in the device tree, add multiple possible states for the `cpu` node and define them in the `power-states` node.

Then, when you use the `k_sleep` function or anything that puts the cpu in idle mode, the thread idle will choose the most adapted idle mode for the idle duration. So you will have to do nothing, and zephyr will do it for you.

You must NEVER perform manual sleep in the main thread, because it can break something else when you use `k_sleep` or an equivalent function.

## Initialization before main

If some device or driver needs to be initialized before the main function, you can use the macro `SYS_INIT(function, level, priority)`. For example, you can configure the clock with it.

## Lora (SX126X)

Define in the device tree the node `sx126x` in the `spi` node using the compatible ` semtech,sx1261` or ` semtech,sx1262`. By default the device sx126x will be put to sleep during the initialization before the main function. The code source of the loramac node is included by default in the Kconfig. You don’t need to do anything else. The device tree binding ` semtech,sx1262` initialize undocumented register to produce a continuous wave, but not the sx1261.

For loraWan use `lorawan_start()` ,  then `lorawan_join()` with all the key needed then you can do downlink or send data to the server. For the join, the Network key is the same as the application key.

## SystemView

Install Systemview and j-link.

Build your zephyr project with the flag `-S rtt-tracing` and flash it

Ex: west build -p always -b nxg_court -S rtt-tracing zephyr_nxg_court

west flash

Open system view and start recording.

In the time line you can see the switch between all the tasks in real time, as well as the interrupt and scheluder, and idle thread.

The event list show all the event track by zephyr by default and the one you may have had with the timestamp and contextual information.

In system view system you can chose the event you want to track or not.

In the windows terminal you can track messages print via the function ` SEGGER_SYSVIEW_PrintfHost()`, ` SEGGER_SYSVIEW_WarnfHost ()`, ` SEGGER_SYSVIEW_ErrorfHost()`

For more detail the complete doc : https://www.segger.com/downloads/jlink/UM08027_SystemView.pdf

To add custom event, you need to edit the "C:\Program Files\SEGGER\SystemView\Description\SYSVIEW_Zephyr.txt". At the end of the file you can had event, and each time increment the event number, change the event name and description. In your code source you can add ` SEGGER_SYSVIEW_RecordXX(id_event, XX)` or `SEGGER_SYSVIEW_RecordEndCallXX(id_event, XX)` to track the beginning and the end of the event if needed. With XX you can choose the number of argument you want to record.

## RTT

Install j-link.

In the Kconfig add the configuration :

CONFIG_USE_SEGGER_RTT=y

CONFIG_LOG_BACKEND_RTT=y		->log using rtt, can use rtt and uart at the same time

CONFIG_RTT_CONSOLE=y			-> work only if uart is disable for the console

CONFIG_UART_CONSOLE=n